8MP HD Intelligent LightHunter Fixed Eyeball Network Camera

IPC3618SB-ADF28(40)KM-I0

Key Features

- High quality image with 8MP, 1/2.7"CMOS sensor

- 8MP (3840*2160)@20fps; 5MP (3072*1728)@30/25fps; 4MP (2560*1440)@30/25fps; 2MP (1920*1080) @30/25fps

- Ultra 265, H.265, H.264, MJPEG

- Intelligent perimeter protection, include cross line, intrusion, enter area, leave area detection

- Based on target classification, smart intrusion prevention significantly reduce false alarm caused by leaves, birds and lights etc., accurately focus on human and, motor vehicle and non-motor vehicle

- Intelligent People Flow Counting and Crowd Density Monitoring

- 120dB true WDR technology enables clear image in strong light scene

- Support 9:16 Corridor Mode

- Built-in Mic

- Smart IR, up to 30m (98ft) IR distance

- Supports 256 G Micro SD card

- IP67 protection

Specifications

|  | IPC3618SB-ADF28KM-I0 |  |  |  |  | IPC3618SB-ADF40KM-I0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |  |  |  |  |
| Sensor | 1/2.7", 8.0 megapixel, progressive scan, CMOS |  |  |  |  |  |  |  |  |
| Minimum Illumination | Colour: 0.0025 lux (F1.6, AGC ON) 0 lux with IR on |  |  |  |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |  |  |  |
| Shutter | Auto/Manual, 1 ~ 1/100000s |  |  |  |  |  |  |  |  |
| Adjustment angle | Pan: 3° ~ 360° |  | Tilt: 0° ~ 80° |  |  |  |  | Rotate: 3° ~ 360° |  |
| WDR | 120dB |  |  |  |  |  |  |  |  |
| S/N | >56dB |  |  |  |  |  |  |  |  |
| Lens |  |  |  |  |  |  |  |  |  |
| Lens | 2.8mm@F1.6 |  |  |  | 4.0mm@F1.6 |  |  |  |  |
| Iris | Fixed |  |  |  |  |  |  |  |  |
| Field of View (H) | 112.4° |  |  |  | 86.5° |  |  |  |  |
| Field of View (V) | 60.1° |  |  |  | 44.1° |  |  |  |  |
| Field of View (D) | 146.1° |  |  |  | 112.5° |  |  |  |  |
| DORI |  |  |  |  |  |  |  |  |  |
| DORI Distance | Lens (mm) | Detect (m) |  | Observe (m) |  |  | Recognize (m) |  | Identify (m) |
|  | 2.8 | 84.0 |  | 33.6 |  |  | 16.8 |  | 8.4 |
|  | 4.0 | 120.0 |  | 48.0 |  |  | 24.0 |  | 12.0 |
| Illuminator |  |  |  |  |  |  |  |  |  |
| IR Range | Up to 30m (98 ft) IR range |  |  |  |  |  |  |  |  |
| Wavelength | 850nm |  |  |  |  |  |  |  |  |
| IR On/Off Control | Auto/Manual |  |  |  |  |  |  |  |  |
| Video |  |  |  |  |  |  |  |  |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |  |  |  |  |  |  |  |  |
| H.264 code profile | Baseline profile, Main profile, High profile |  |  |  |  |  |  |  |  |
| Frame Rate | Main Stream: 8MP (3840*2160), Max 20fps; 5MP (3072*1728), Max 30fps; 4MP (2560*1440), Max 30fps; 1080P (1920*1080), Max 30fps; Sub Stream: 1080P (1920*1080), Max 30fps;720P (1280*720), Max 30fps; D1 (720*576), Max 30fps; 640*360,Max 30fps; 2CIF(704*288), Max 30fps; CIF(352*288), Max 30fps; Third Stream: D1 (720*576), Max 30fps; 640*360,Max 30fps; 2CIF(704*288), Max 30fps; CIF(352*288), Max 30fps; |  |  |  |  |  |  |  |  |
| Video Bit Rate | 128 Kbps~16 Mbps |  |  |  |  |  |  |  |  |
| U-code | Support |  |  |  |  |  |  |  |  |
| OSD | Up to 8 OSDs |  |  |  |  |  |  |  |  |
| Privacy Mask | Up to 4 areas |  |  |  |  |  |  |  |  |
| ROI | Up to 8 areas |  |  |  |  |  |  |  |  |
| Video Stream | Triple streams |  |  |  |  |  |  |  |  |
| Image |  |  |  |  |  |  |  |  |  |
| White Balance | Auto/Outdoor/Fine Tune/Sodium Lamp/Locked/Auto2 |  |  |  |  |  |  |  |  |
| Image Setting | Image rotation, brightness, saturation, contrast, sharpness, gain adjustable by web browser |  |  |  |  |  |  |  |  |
| Digital Noise Reduction | 2D/3D DNR |  |  |  |  |  |  |  |  |
| Smart IR | Support |  |  |  |  |  |  |  |  |
| Flip | Normal/Vertical/Horizontal/180°/90°Clockwise/90°Anti-clockwise |  |  |  |  |  |  |  |  |
| Dewarping | Support |  |  |  |  |  |  |  |  |
| HLC | Support |  |  |  |  |  |  |  |  |
| BLC | Support |  |  |  |  |  |  |  |  |
| Defog | Digital Defog |  |  |  |  |  |  |  |  |
| Intelligent |  |  |  |  |  |  |  |  |  |
| Smart intrusion prevention | Support false alarm filtering: Cross Line detection, Intrusion detection, Enter Area detection, Leave Area detection |  |  |  |  |  |  |  |  |
| Statistical Analysis | People counting, support people flow counting and crowd density monitoring |  |  |  |  |  |  |  |  |
| Events |  |  |  |  |  |  |  |  |  |
| Basic Detection | Motion detection, Ultra motion detection, Tampering alarm, Audio detection |  |  |  |  |  |  |  |  |
| General Function | Watermark, IP address filtering, tampering alarm, access policy, ARP protection, RTSP authentication, user authentication |  |  |  |  |  |  |  |  |
| Audio |  |  |  |  |  |  |  |  |  |
| Audio Compression | G.711 |  |  |  |  |  |  |  |  |
| Audio Bitrate | 64 Kbps |  |  |  |  |  |  |  |  |
| Suppression | Support |  |  |  |  |  |  |  |  |
| Sampling Rate | 8KHZ |  |  |  |  |  |  |  |  |
| Storage |  |  |  |  |  |  |  |  |  |
| Edge Storage | Micro SD, up to 256GB |  |  |  |  |  |  |  |  |
| Network Storage | ANR, NAS(NFS) |  |  |  |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |  |  |
| Protocols | IPv4, IPv6, IGMP, ICMP, ARP, TCP, UDP, DHCP, PPPoE, RTP, RTSP, RTCP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, 802.1x, SNMP, QoS, RTMP,SSL/TLS, SSH |  |  |  |  |  |  |  |  |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T), API,SDK |  |  |  |  |  |  |  |  |
| User/Host | Up to 32 users. 3 user levels: administrator, common user, operator |  |  |  |  |  |  |  |  |
| Security | Password protection, Strong password, HTTPS encryption, Export operation logs, Basic and digest authentication for RTSP, Digest authentication for HTTP, TLS 1.2, WSSE and digest authentication for ONVIF |  |  |  |  |  |  |  |  |
| Client | EZStation EZView EZLive |  |  |  |  |  |  |  |  |
| Web Browser | Plug-in required live view: IE 10 and above, Chrome 45 and above, Firefox 52 and above, Edge 79 and above |  |  |  |  |  |  |  |  |
|  | Plug-in free live view: Chrome 57.0 and above, Firefox 58.0 and above, Edge 16 and above |  |  |  |  |  |  |  |  |
| Interface |  |  |  |  |  |  |  |  |  |
| Audio I/O | N/A |  |  |  |  |  |  |  |  |
| Alarm I/O | N/A |  |  |  |  |  |  |  |  |
| RS485 | N/A |  |  |  |  |  |  |  |  |
| Built-in Mic | Support |  |  |  |  |  |  |  |  |
| Built-in Speaker | N/A |  |  |  |  |  |  |  |  |
| WIFI | N/A |  |  |  |  |  |  |  |  |
| Network | 1 * RJ45 10M/100M Base-TX Ethernet |  |  |  |  |  |  |  |  |
| Video Output | N/A |  |  |  |  |  |  |  |  |
| Certifications |  |  |  |  |  |  |  |  |  |
| EMC | CE-EMC (EN 55032: 2015+A1:2020,EN 61000-3-3: 2013+A1: 2019,EN IEC 61000-3-2: 2019+A1: 2021,EN 55035: 2017+A11:2020) FCC (FCC CFR 47 part15 B, ANSI C63.4-2014) |  |  |  |  |  |  |  |  |
| Safety | CE LVD (EN 62368-1:2014+A11:2017) CB (IEC 62368-1:2014) UL (UL 62368-1, 2nd Ed., Issue Date: 2014-12-01) |  |  |  |  |  |  |  |  |
| Environment | CE-RoHS (2011/65/EU;(EU)2015/863); WEEE (2012/19/EU); Reach (Regulation (EC) No 1907/2006) |  |  |  |  |  |  |  |  |
| Protection | IP67 (IEC 60529:1989+AMD1:1999+AMD2:2013) |  |  |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |  |  |
| Power | DC 12V±25%, PoE (IEEE 802.3af) |  |  |  |  |  |  |  |  |
|  | Power consumption: Max 5.0W |  |  |  |  |  |  |  |  |
| Power Interface | Ø 5.5mm coaxial power plug |  |  |  |  |  |  |  |  |
| Dimensions (L × W × H) | Φ110.5 x 94.1mm (Φ4.4” x 3.7”) |  |  |  |  |  |  |  |  |
| Weight | 0.44kg (0.97lb) |  |  |  |  |  |  |  |  |
| Material | Metal |  |  |  |  |  |  |  |  |
| Working Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |  |
| Strorage Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |  |
| Surge Protection | 4KV |  |  |  |  |  |  |  |  |
| Reset Button | N/A |  |  |  |  |  |  |  |  |

Dimensions

Accessories

| TR-JB03-G-IN | TR-JB07/WM03-G-IN | TR-WM03-D-IN | TR-UM06-E-IN |
| --- | --- | --- | --- |
| Junction Box | Wall Mount | Wall Mount | Incline Ceiling Mount |
| TR-UP06-IN | TR-A01-IN | TR-SE24-IN | TR-SE24-A-IN |
| Pole Mount | Waterproof Joint | Pendant Mount | Pendant Mount |
| TR-CM24-IN |  |  |  |
| Pendant Mount |  |  |  |

| Junction Mount | Pole Mount |
| --- | --- |
| TR-JB03-G-IN | TR-JB07/WM03-G-IN |
| TR-UM06-E-IN | TR-JB07/WM03-G-IN+TR-UP06-IN |
| TR-CM24-IN+TR-JB03-G-IN |  |

Zhejiang Uniview Technologies Co., Ltd.

Building No.10, Wanlun Science Park, Jiangling Road 88, Binjiang District, Hangzhou, Zhejiang, China (310051)

Email: overseasbusiness@uniview.com; globalsupport@uniview.com

http://www.uniview.com

©2020-2022 Zhejiang Uniview Technologies Co., Ltd. All rights reserved.

*Product specifications and availability are subject to change without notice.